3.Field-Effect Transistor (FET)

Junction Field-Effect Transistor (JFET)

n-channel JFET

VGS = 0 V and VDS positive value

When VGS=0V and C some positive value,the upper region of the p-type material will be reverse biased higher than the lower region(the greater the reverse bias,the wider the depletion region).

As the voltage VDS is increased from 0 to a few volts,the current will increase as determined by Ohm’s law and The depletion region will widen,causing a noticeable reduction in channel widht.If VDS is increased to a level where it appears that the two depletion regions would “touch”, a condition referred to as pinch-off will result.The level of VDS that establishes this condition is referred to as pinch-off voltage Vp (when VGS=0V and VDS>Vp IDSS).

Input resistance >

VGS < 0V

Voltage-Controlled Resistor:

where ro is the resistance with VGS _ 0 V and rd the resistance at a particular level of VGS.

Power dissipation:

p-channel JFET

PD = |VD|ID

Metal-Oxide-Semiconductor Field-Effect Transistor (MOSFET) Or Insulated-Gate FET (IGFET)

Depletio-Type MOSFET (D-MOSFET) n-channel D-MOSFET

When VGS=0V,the drain current is IDSS.

The negative potential at the gate will tend to pressure electrons toward the p-type substrate and attract holes from the p-type substrate.The drain current is therefore reduce with increasing negative bias for VGS.

The positive potential at the gate will draw additional electrons (free carrier) from the p-type substrate due to the reverse leakage current,reveals that the drain current will increase.

Input resistance  >

p-channel D-MOSFET

Enhachement-Type MOSFET (E-MOSFET)

n-channel E-MOSFET

If  VGS = 0V,the absence of an n-channel will result in a

current of effectively zero amperes.

p-channel E-MOSFET

Complementary MOSFET (CMOS)

A very effective logic circuit can be established by contructing a p-channel and an n-channel MOSFET on the same substrate.

FET Biasing

For de analysis,the coupling capacitors are open cicuits.

Fixed-Bias Configuration

Graphical Approach:

Self-Bias Configuration		Graphical Approach:

Voltage-Divinder Biasing									Graphical Approach:

Depletion-Type MOSFET

Graphical Approach:

Enhancement-Type MOSFET

Graphical Approach:

p-channel FET

Graphical Approach:

FET Small-Signal Model

Input Impedance of the FET:

(the typical values: JFET

Output impedance of the FET:

FET Small-Signal Analysis

Common-Source Circuit

Common-Drain (Source-Follower) Circuit

Common-Gate Circuit

Enhancemen  MOSFET Amplifier

(for

(for or

System Approach-Effects of RS and RL

Bypassed Source Resistance

Unbypassed Source Resistance

Source Follower

Common Gate